C:\PC_App_Projects\DDASPostProc5\Process\Debug>process

DDAS Data Processor ---- Version 5.00  Built Apr 28 2014

by Instrumentation Engineering

Copyright (c) 1994-2014 Chrysler Group LLC

Performs mathematical operations on DDAS processed data.

Usage:  process infile operator operand [title1 [title2]...[/option]]

infile       - processed data file from DDAS (*.fpd)

operator     - * multiply, ^ raise to power, + add, or - subtract

operand      - constant or 2nd data file.  (power (^) requires constant)

title1, title2 - first and subsequent words of output title

- Required if 'infile' name is channel number (CH####.fpd)

option       - /E=engineering units

/V=verbose

Examples:    process ch0119 + ch0120 Pendulum Accel Sum

or process ch0119 + ch0120 "Pendulum Accel Sum"

add channels 19 and 20 of unit 1

and store in "Pendulum Accel Sum.fpd"

process "Pendulum Accel Sum" * 0.5

multiply data in "Pendulum Accel Sum.fpd" by 0.5

and store in "Pendulum Accel Sum.fpd"

NOTE: If infile name, operand name or engineering units contain blanks,

enclose the text in double quotes ["]

ERROR DETECTED,  Any key terminates program...